Date: Supercedes:

15 October 2001 08 March 2000

MATERIAL SAFETY DATA SHEET

IN CASE OF EMERGENCY CALL CHEMTREC AT 1-800-424-9300

PRODUCT IDENTIFICATION AND COMPANY IDENTIFICATION:

Product Name:

PURELL® INSTANT HAND SANITIZER DRY HANDS FORMULA

Company Name & Address:

Emergency Phone:

Non-Emergency Phone: MSDS Request Phone:

GOJO Industries, Inc.

One GOJO Plaza, Suite 500

Akron, OH 44311

1-800-424-9300 CHEMTREC

(330) 255-6000 (330) 255-6000 x8804

INFORMATION ON INGREDIENTS:

| HAZARDOUS INGREDIENTS | CAS NUMBER | OSHAPEL | ACGIH TLV | % RANGE |
| --- | --- | --- | --- | --- |
| Ethyl Alcohol | 64-17-5 | 1000ppm | 1000 ppm | 62 |
| Isopropanol | 67-63-0 | 400 ppm | 400 ppm | <5 |
| Glyerin, as mist | 56-81-5 | 15 mg/mj | 10mg/mJ | <3 |

| Other ingredient(s) with notification requirements: | CAS NUMBER | List |  |
| --- | --- | --- | --- |
| Ethyl Alcohol | 64-17-5 | MA1;NJ 1S;PA1; | CN2 |
| Isopropanol | 67-63-0 | MA1;NJ 1S;CN1 |  |
| Glyerin, as mist | 56-81-5 | MA1; PA1 |  |

HAZARDS IDENTIFICATION:

EMERGENCY OVERVIEW

When used according to instructions, the product applicable to this MSDS is safe and presents no immediate or long-term health hazard. However, abnormal entry routes, such as gross ingestion, may require immediate medical attention.

Potential Health Effects:

HMIS:

Eye Contact: Skin Contact: Inhalation: Ingestion: Carcinogenicity:

Health     1   Flammability     3   Reactivity     0   Personal Protection   None

May cause eye irritation.

No irritation or reaction expected.

Not applicable.

May cause upset stomach, nausea (Abnormal entry route).

Not listed as a carcinogen by NTP, IARC, OSHA or ACGIH.

4.

FIRST AID MEASURES:

Eye Contact:

Skin Contact:

Inhalation:

Ingestion:

Do not rub eyes. Flush eyes thoroughly with water for 15 minutes. If condition

worsens or irritation persists, contact physician.

Not applicable.

Not applicable.

Do not induce vomiting. Contact a physician or Poison Control Center.

JELL® INSTANT HAND SANITIZER DRY HANDS FORMULA	Page 2 of 2

FIRE FIGHTING MEASURES:

NFPA:	Health        0   Fire     3   Reactivity     0

Flashpoint °F/°C (PMCC method):       74°F/23°C

Unusual Fire and Explosion Hazards:  Product is flammable due to alcohol content.

Special Fire Fighting Procedures:         None known.

Extinguishing Media:    X  Water Fog    X   Alcohol Foam   X    CO2    X   Dry Chemical	Other

ACCIDENTAL RELEASE MEASURES:

Avoid contact with ignition sources since product is flammable. Absorb onto inert material and dispose in appropriate manner. Water clean up and rinse. CAUTION -WILL CAUSE SLIPPERY SURFACES.

HANDLING AND STORAGE:

Keep away from fire or flame. Store at normal room temperature away from reach of small children. Keep containers sealed. Use older containers first. Avoid freezing conditions.

EXPOSURE CONTROLS/PERSONAL PROTECTION:

Eye Protection:	None required under normal conditions.

Skin Protection:	None required under normal conditions.

Respiratory Protection:	None required under normal conditions.

Ventilation:	None required under normal conditions.

Protective Equipment or Clothing:	None required under normal conditions.

PHYSICAL AND CHEMICAL PROPERTIES:

Appearance and Odor	White opaque liquid, herbal fruit fragrance

pH (undiluted):	7 - 8.25

VOC , %:	65

STABILITY AND REACTIVITY:

Stable/Non reactive product.   Avoid ignition sources.

TOXICOLOGICAL INFORMATION:

No acute or chronic toxic effects expected when used according to directions.

ECOLOGICAL CONSIDERATIONS:

No ecological or special considerations when used according to directions. Not considered environmentally harmful from normal dilution, expected usage and typical drainage to sewers, septic systems and treatment plants.

DISPOSAL CONSIDERATIONS:

Characteristic hazardous waste-flammable liquid. Dispose according to local, state and Federal regulations. TRANSPORT INFORMATION:

Hazardous by transport regulations. When transported by Ground and Rail, this product typically is shipped as Consumer Commodity ORM-D. When transported by air, this product is typically shipped as Consumer Commodity or Alcohols N.O.S. depending on package size. When transported by ocean, this product is typically shipped as Limited Quantities. Refer to current regulations for exact requirements.

REGULATORY AND OTHER INFORMATION:

Complies with current FDA regulations for cosmetic and/or over-the-counter drug products. Notice: The information herein is based on data considered to be accurate as of the date of preparation of this material safety data sheet. However, no warranty or representation, expressed or implied, is made as to the accuracy or completeness of the foregoing data and safety information. The user assumes all liability for any damage or injury resulting from abnormal use, from any failure to adhere to recommended practices or from any hazards inherent in the nature of the product.

ment#: 9652-503